# Dynamic line-pack dispatch under TOU pricing: physics-faithful CMDP, constraint-aware DRL, MPC distillation, and a two-network benchmark

## Summary

This PR extends the steady-state DRL pipeline-optimisation framework of Liu et al. (Energy & AI, 2024) into a dynamic, economically-driven dispatch problem and builds a complete, fair, reproducible study around it. It turns the project from a multi-algorithm sandbox into a coherent benchmark with a Markdown/Word paper draft (paper/draft.md, paper/draft.docx).

The problem: an electric-driven compressor can shift compression work in time, using the pipeline’s line-pack as a short-term store to exploit time-of-use (TOU) electricity prices, subject to node-pressure, compressor speed and surge/choke constraints and an end-of-day inventory target.

## Key contributions

- Physics-faithful dynamic CMDP. A 24-hour line-pack environment with TOU economics that keeps the paper’s centrifugal-compressor maps (adiabatic head, head/efficiency polynomials, Eqs. 7–11) and encodes the full operating envelope (Eq. 20) as state-dependent constraints. A single source-of-truth compressor module (envs/compressor.py) is shared by the environment and every planner, so all methods are scored against identical physics.

- Feasibility-aware action realisation (the key enabler). The compressor’s feasible set is disconnected (off, or on above a state-dependent minimum speed). A continuous policy otherwise keeps commanding the infeasible “dead band” and incurs ~6 envelope violations/day regardless of training budget. Realising dead-band commands as the unit staying off makes the action map onto the feasible set and takes every DRL agent to zero envelope violations.

- Constrained (Lagrangian) SAC. Optimises economics subject to a single, weight-free, normalised constraint cost, with a dual variable adapted by projected dual ascent — removing hand-tuned penalty weights.

- MPC distillation closes the cost premium. From-scratch DRL is feasible but ~2–3× the optimum cost. Distilling the MPC feedback law into a small MLP (behavioral cloning + DAgger) yields a policy that matches MPC cost at ~ms latency (the from-scratch premium is removed).

- Three-network benchmark. Rule-based, GA, DP, receding-horizon MPC, SAC/TD3/PPO, Constrained-SAC and Distilled-MPC, all on one objective, with multi-seed statistics, robustness under perturbation, and an honest discussion. Validated on (i) a gun-barrel element, (ii) a branched benchmark, and (iii) a sub-network with native pipe geometry from the real GasLib-40 instance; the env/DP/MPC/distillation are all generalised to arbitrary single-compressor topologies (N-D DP).

## Results (real, reduced-but-real: 150k steps, 3 seeds, 6 perturbed scenarios)

Gun-barrel network — 24-h electricity cost / violation-hours / latency:

| Method | Cost | Viol-h | Latency |
| --- | --- | --- | --- |
| GA / DP / MPC (near-optimal refs) | 4.3–5.4 | 0 | 1–11 s |
| Distilled-MPC | 5.34 | ~0.7 | 2.8 ms |
| SAC / Constrained-SAC (from scratch) | 15.0 / 13.4 | 0 | ~5 ms |
| Rule-based | 22.9 | 19 | – |

Branched benchmark — the core findings transfer; the cost premium shrinks:

| Method | Cost | Viol-h | Term-gap | Latency |
| --- | --- | --- | --- | --- |
| GA (clean reference) | 5.94 | 0 | 0 | 11.5 s |
| SAC | 4.46 | 0 | 318 | 4.6 ms |
| Distilled-MPC | 6.21 | 2.7 | 11 | 3.3 ms |
| DP / MPC (3-D grid) | 6.02 | 2.0 | 128 | 6.8 s |
| Constrained-SAC | 14.40 | 0.3 | 357 | 4.6 ms |

GasLib-40-derived network (real pipe geometry) — the cleanest case for the method:

| Method | Cost | Viol-h | Term-gap | Latency |
| --- | --- | --- | --- | --- |
| GA (optimum ref) | 5.57 | 0 | 5 | 11.9 s |
| Distilled-MPC | 5.26 | 0 | 111 | 2.8 ms |
| DP / MPC | 6.70 | 0 | 141 | 6.5 s |
| Constrained-SAC | 17.08 | 0 | 66 | 4.7 ms |
| SAC | 16.07 | 0 | 300 | 4.9 ms |

On the real-geometry network the distilled controller is the best learned method outright (matches the GA optimum, beats MPC, 0 violations, ms latency).

Honest caveats (in the paper): the constraint-aware-SAC ranking is network-dependent (it wins on the gun-barrel and GasLib nets, loses on the branched one); the branched 3-D grid DP/MPC are grid-limited (~2 violations) so GA is the clean reference there; TD3 degenerates (never compresses → depletes line-pack), which the terminal-gap column exposes; the GasLib geometry needed affine recalibration for numerical stability and the TOU price stays synthetic (GasLib is gas-only).

## What’s in this PR

- envs/compressor.py — shared compressor physics (Eqs. 7–11) + feasibility-aware action realisation.

- envs/pipeline_env.py — topology-general dynamic line-pack CMDP; branched_benchmark_config(), gaslib_benchmark_config().

- DP.py — DP reference (2-D fast path + general N-D); baselines.py — rule / GA / MPC + evaluators.

- constrained_sac.py — Lagrangian SAC; distill_mpc.py — BC + DAgger MPC distillation.

- run_experiments.py — unified harness (--network {default,branched}, multi-seed, figures + CSVs).

- paper/draft.md, paper/draft.docx — the paper draft; paper/make_table.py — table generator.

- Results: models/exports/paper/ (gun-barrel), models/exports/paper_branched/, models/exports/paper_gaslib/; all raw data bundled in raw_data/ (+ raw_data.zip).

## Reproduce

python run_experiments.py --quick                              # fast plumbing check
python run_experiments.py --steps 150000 --seeds 3 --perturb 6 # gun-barrel
python run_experiments.py --network branched --steps 150000 --seeds 3 --dagger 12
python run_experiments.py --network gaslib   --steps 150000 --seeds 3 --dagger 12  # GasLib-40 geometry
python paper/make_table.py                                     # regenerate the results table

## Status

Reduced-but-real budget (150k steps / 3 seeds) — numbers are genuine but not camera-ready. The remaining credibility lift is a field / GasLib-derived network with measured load and price; the branched benchmark is a step toward that.